# parametric upper limb and hand 3d print prosthetic product design via anthropometic data and artificial inteligence

Research shows that parametric design approaches combining anthropometric data (such as grip circumference and palm breadth) with AI methods can generate personalized 3D-printed upper limb prostheses with proven functional performance.

## Abstract

Ten  studies  demonstrate  that  parametric  techniques  can  tailor  upper  limb  and  hand  prostheses  using  anthropometric  data  and,  in  some  cases,  artificial  intelligence.  Four  studies  detail  the  use  of  specific  measurements—such  as  grip  circumference,  palm  breadth,  thumb  dimensions,  and  wrist  range  of  motion—to  generate  anatomically  personalized  designs  via  Computer-Aided  Design  protocols  (e.g.,  Solidworks  and  Blender).  Two  studies  employ  AI  or  data‐driven  methods:  one  classifies  electromyographic  signals  for  adaptive  control  (yielding  a  7.4/10  BAM-ULA  score  with  100%  gross  motor  task  success)  and  another  applies  multi-index  optimization  for  socket  design.

Manufacturing  methods  predominantly  rely  on  three-dimensional  printing  techniques—fused  deposition  modeling,  selective  laser  sintering,  and  desktop  printing—with  materials  including  PLA,  ABS,  and  nylon.  Reported  costs  range  from  low  material  expenditures  (approximately  US$50)  to  complete  devices  under  US$350.  Target  populations  span  transradial  amputees,  children,  and  users  in  low-income  or  developing  regions.  These  studies  indicate  that  parametric  design  supported  by  anthropometric  data—and,  when  applied,  selective  AI  approaches—can  produce  adjustable,  cost-effective  prosthetic  solutions  with  quantifiable  functional  outcomes.

## Paper search

We performed a semantic search using the query "parametric upper limb and hand 3d print prosthetic product design via anthropometic data and artificial inteligence" across over 138 million academic papers from the Elicit search engine, which includes all of Semantic Scholar and OpenAlex.

We retrieved the 50 papers most relevant to the query.

## Screening

We screened in sources that met these criteria:

- Upper Limb Prosthetics Focus: Does this study focus on upper limb prosthetics (including partial hand, wrist, forearm, or full arm prosthetics)?

- 3D Printing Technology: Does this study involve 3D printing/additive manufacturing technology for prosthetic production?

- Parametric Design Approach: Does this study incorporate parametric design approaches in prosthetic development?

- Anthropometric Data Utilization: Does this study utilize anthropometric data in the prosthetic design process?

- AI/ML Integration: Does this study employ artificial intelligence, machine learning, or automated design algorithms in prosthetic development?

- Human Validation: Does this study involve human participants (amputees, prosthetic users) or include design validation with the target population?

- Relevant Outcomes: Does this study report on design methodology, functional outcomes, fit assessment, or user satisfaction?

- Methodological Rigor: Is this study a full research article with complete methodology (not an opinion piece, editorial, or conference abstract without full methodology)?

We considered all screening questions together and made a holistic judgement about whether to screen in each paper.

## Data extraction

We asked a large language model to extract each data column below from each paper. We gave the model the extraction instructions shown below for each column.

- Parametric Design Approach:

- Extract details about the parametric design methodology including:

- Which anthropometric parameters were used (e.g., palm breadth, grip circumference, hand length)

- How parameters were selected or validated (statistical analysis, biomechanical studies)

- Parameter relationships and dependencies (which parameters drive others)

- Number of design variables and their ranges

- Parametric modeling software or methods used

- Whether design scales automatically or requires manual adjustment

- Anthropometric Data Collection:

- Extract methods for obtaining body measurements including:

- Data collection technique (3D scanning, photography, manual measurement, existing databases)

- Equipment used (smartphones, 3D scanners, measuring tools)

- Processing algorithms (Statistical Shape Models, computer vision, manual digitization)

- Accuracy/error rates of measurement methods

- Sample size and demographics of anthropometric data

- Which body segments or features were measured

- AI Implementation:

- Extract details about artificial intelligence integration including:

- AI application area (design optimization, control systems, customization, measurement processing)

- Machine learning algorithms used (classification, regression, neural networks)

- Input data types (EMG signals, images, sensor data)

- Training data and methods

- AI performance metrics (accuracy, response time, success rates)

- Whether AI learns/adapts to individual users

- 3D Printing Specifications:

- Extract 3D printing technical details including:

- Printing technology (FDM, SLA, SLS, material extrusion)

- Materials used and their properties

- Print settings and parameters

- Assembly requirements (single print vs multiple parts)

- Post-processing steps required

- Print time and material costs

- Printing challenges and solutions identified

- Functional Performance:

- Extract quantitative performance results including:

- Grip/pinch force capabilities (N or kg)

- Range of motion measurements

- Control accuracy and response time

- Battery life or power consumption

- Standardized assessment scores (BAM-ULA, DASH, etc.)

- Comparison to commercial prosthetics or benchmark values

- Durability testing results

- User Testing Results:

- Extract user evaluation outcomes including:

- Number and demographics of test participants

- Testing protocol or tasks performed

- User satisfaction ratings or feedback

- Ease of use and learning curve

- Daily living activity performance

- User preferences for design features

- Acceptance rates and reasons for rejection/adoption

- Design Principles:

- Extract key design considerations including:

- Target user population (age, amputation level, geographic region)

- Cost targets and actual costs achieved

- Manufacturing simplicity requirements

- Modularity and customization capabilities

- Accessibility features for low-resource settings

- Bio-inspired design elements incorporated

- Trade-offs between functionality and simplicity

# Results

## Characteristics of Included Studies

| Study | Study Focus | Design Approach | Manufacturing Method | Target Population | Full text retrieved |
| --- | --- | --- | --- | --- | --- |
| Romero et al., 2025 | Artificial intelligence-driven, three-dimensional printed myoelectric prosthesis with anatomical personalization | Parametric modeling, artificial intelligence-based control | Additive manufacturing (three-dimensional printing), three-dimensional scanning | Transradial amputees, low-income regions | No |
| Lopez, 2021 | Accessible, three-dimensional printed hand prostheses for low-income settings | Parametric design, non-assembly fabrication, bio-inspired | Material extrusion (fused deposition modeling), smartphone-based three-dimensional modeling | Transradial amputees, low-income/developing countries | No |
| Moreo, 2016 | Parametric, body-powered three-dimensional printed hand for children | Parametric design, statistical analysis of anthropometry | Three-dimensional printing (no mention found), modular assembly | Children in developing countries | No |
| Saldarriaga et al., 2024 | Parametric design for affordable, customized three-dimensional printed sockets | Parametric design, customization | Three-dimensional printing with polylactic acid (PLA) | Transradial congenital amputees | No |
| Fajardo et al., 2020 | Galileo Hand: modular, parametric, three-dimensional printed myoelectric prosthesis | Modular, parametric, self-contained, user feedback | Three-dimensional printing (nylon, acrylonitrile butadiene styrene (ABS), polylactic acid (PLA)), multi-part assembly | Transradial amputees, developing countries | Yes |
| Nini et al., 2024 | Parametric computer-aided design (CAD) for custom prosthetic fingers | Parametric CAD, anthropometric modeling | Selective laser sintering (SLS) three-dimensional printing, three-dimensional scanning | Adults (tested on one volunteer) | No |
| Zuniga et al., 2015 | Cyborg Beast: low-cost, three-dimensional printed hand for children | Parametric scaling, distance fitting, open-source | Fused deposition modeling (PLA, ABS), multi-part assembly | Children with upper-limb reductions | Yes |
| Fajardo et al., 2017 | Open-source, multifunctional, modular three-dimensional printed prosthesis | Modular, parametric, self-contained, surface electromyography (sEMG) control | Three-dimensional printing (no mention found) | Transradial amputees (various levels) | No |
| Cabibihan et al., 2018 | Patient-specific prosthetic arms via computed tomography (CT), computer-aided design (CAD), three-dimensional printing | Mirrored geometry, high-accuracy fit | Desktop three-dimensional printing, silicone casting | Upper limb amputees (no further specification) | No |
| Gu et al., 2024 | Data-driven, multi-index optimization for socket design | Data-driven, multi-index optimization, clinical criteria | No mention found | Forearm amputees | No |

### Design Approaches

- Parametric  design/modeling/scaling :  Found  in  8  studies.

- Modularity :  Used  in  2  studies.

- Artificial  intelligence  or  data-driven  approaches :  Used  in  2  studies.

- Anthropometric  or  statistical  modeling :  Used  in  2  studies.

- Customization :  Focus  in  2  studies.

- Other  approaches :  Open-source,  user  feedback,  surface  electromyography/artificial  intelligence  control,  non-assembly,  and  bio-inspired  approaches  each  found  in  1  study.

### Manufacturing Methods

- Three-dimensional  printing  (general  or  no  mention  found) :  Used  in  6  studies.

- Fused  deposition  modeling/material  extrusion :  Used  in  2  studies.

- Selective  laser  sintering :  Used  in  1  study.

- Three-dimensional  scanning :  Used  in  2  studies.

- Modular  or  multi-part  assembly :  Used  in  2  studies.

- Materials :  Polylactic  acid  (PLA)  specified  in  3  studies,  acrylonitrile  butadiene  styrene  (ABS)  in  2,  nylon  in  1.

- Other  methods :  Smartphone-based  and  desktop  three-dimensional  printing  each  used  in  1  study;  silicone  casting  in  1  study.

- No  mention  found :  Manufacturing  method  not  described  in  1  study.

### Target Populations

- Transradial  amputees  (including  forearm) :  Target  in  6  studies.

- Children :  Target  in  2  studies.

- Low-income  or  developing  regions/countries :  Focus  in  4  studies.

- Congenital  amputees :  Target  in  1  study.

- Adults :  Target  in  1  study  (tested  on  a  single  volunteer).

- Upper  limb  amputees  (no  further  specification) :  1  study.

- All  studies  provided  some  population  information.

## Thematic Analysis

### Parametric Design Methodologies

| Study | Parametric Design Details |
| --- | --- |
| Romero et al., 2025 | Uses parametric modeling for anatomical personalization; no mention found of specific parameters or software |
| Lopez, 2021 | Parametric socket design from two-dimensional photos using statistical shape model (SSM); manual adjustment required due to measurement errors |
| Moreo, 2016 | Four key anthropometric parameters (grip circumference, palm breadth, thumb breadth, palm length) selected via statistical analysis; all linked to palm breadth; Solidworks computer-aided design (CAD) with external file for easy adjustment |
| Saldarriaga et al., 2024 | Parametric and transradial prosthesis design; no mention found of specific parameters or software |
| Fajardo et al., 2020 | Modular, parametric design allowing modification of phalanges' length; validated via user feedback; no mention found of software |
| Nini et al., 2024 | Parametric computer-aided design (CAD) for finger design based on anthropometric parameters from three-dimensional scans; no mention found of software |
| Zuniga et al., 2015 | Parametric scaling using hand/forearm length, palm width, wrist range of motion from photos; validated against direct measurement; Blender 7.2 and sizing chart for scaling |
| Fajardo et al., 2017 | Modular, parametric design; no mention found of specific parameters or software |
| Cabibihan et al., 2018 | Mirrored geometry from computed tomography (CT) scans; no mention found of software; focus on high-accuracy fit |
| Gu et al., 2024 | Data-driven, multi-index optimization; no mention found of specific parameters or software |

Anthropometric  Parameters:

- Four  studies  (Moreo,  Fajardo  2020,  Nini,  Zuniga)  detailed  specific  anthropometric  parameters,  most  commonly  grip  circumference,  palm  breadth,  thumb  breadth,  palm  length,  hand/forearm  length,  palm  width,  and  wrist  range  of  motion.

- Five  studies  (Romero,  Lopez,  Saldarriaga,  Fajardo  2017,  Gu)  did  not  mention  specific  parameters.

Software:

- Only  two  studies  specified  the  software  used  for  parametric  design  (Moreo:  Solidworks;  Zuniga:  Blender).  The  other  eight  studies  did  not  mention  software.

Input  Methods:

- Two  studies  used  two-dimensional  photos  (Lopez,  Zuniga),  one  used  three-dimensional  scans  (Nini),  and  one  used  computed  tomography  (CT)  scans  (Cabibihan).

Statistical  or  Data-Driven  Approaches:

- Two  studies  (Lopez,  Gu)  used  statistical  or  data-driven  approaches,  including  statistical  shape  modeling.

Modular  or  Adjustable  Designs:

- Three  studies  (Moreo,  Fajardo  2020,  Fajardo  2017)  described  modular  or  easily  adjustable  designs.

Validation:

- Two  studies  (Fajardo  2020,  Zuniga)  reported  validation  of  the  parametric  design  via  user  feedback  or  direct  measurement.

Other  Notable  Approaches:

- One  study  (Cabibihan)  focused  on  high-accuracy  fit.

- One  study  (Romero)  mentioned  anatomical  personalization.

### Anthropometric Data Integration and Customization

| Study | Data Collection & Processing | Accuracy/Validation | Customization Approach |
| --- | --- | --- | --- |
| Romero et al., 2025 | Three-dimensional scanning; parametric modeling | No mention found | Anatomical personalization |
| Lopez, 2021 | Smartphone photos, statistical shape model (SSM), manual measurement | Automatic method error too large; manual method used | Parametric socket design |
| Moreo, 2016 | No mention found; parameters identified | No mention found | Parametric design for children |
| Saldarriaga et al., 2024 | Anthropometric measurements (no mention found of method) | No mention found | Custom socket for one user |
| Fajardo et al., 2020 | User feedback from volunteers | No mention found | Modular, customizable design |
| Nini et al., 2024 | Three-dimensional scanning of phalanges | No mention found | Custom finger design |
| Zuniga et al., 2015 | Direct measurement, photography, ImageJ | No significant difference between methods; validated | Parametric scaling, distance fitting |
| Fajardo et al., 2017 | No mention found | No mention found | Modular, parametric design |
| Cabibihan et al., 2018 | Computed tomography (CT) imaging, computer-aided design (CAD) | High accuracy (Dice similarity coefficient 0.95–0.96, less than 1% error) | Mirrored geometry for fit |
| Gu et al., 2024 | No mention found; motion/pressure data | No mention found | Data-driven socket optimization |

Data  Collection  &  Processing:

- Three-dimensional  scanning:  2  studies.

- Imaging  (computed  tomography):  1  study.

- Photography  (including  smartphone  photos):  2  studies.

- Direct  or  manual  measurement  (including  anthropometric):  3  studies.

- User  feedback:  1  study.

- Statistical  shape  modeling:  1  study.

- Motion/pressure  data:  1  study.

- No  mention  found  of  data  collection  or  processing  method  in  3  studies.

Accuracy/Validation:

- High  accuracy  or  validation:  2  studies.

- Error  too  large  for  automatic  methods,  manual  methods  used:  1  study.

- No  mention  found  of  accuracy  or  validation  in  7  studies.

Customization  Approach:

- Parametric  design  (including  modeling,  scaling,  or  socket  design):  4  studies.

- Anatomical  personalization  (including  mirrored  geometry):  2  studies.

- Modular  or  customizable  design:  2  studies.

- Data-driven  optimization:  1  study.

- Customization  for  one  user  (including  custom  finger  design):  2  studies.

### Three-Dimensional Printing Technologies and Manufacturing Constraints

| Study | Printing Technology | Materials | Assembly/Post-processing | Cost/Accessibility |
| --- | --- | --- | --- | --- |
| Romero et al., 2025 | No mention found | No mention found | No mention found | Affordable, accessible |
| Lopez, 2021 | Material extrusion (fused deposition modeling) | Three-dimensional printed polymers | Single print, minimal post-assembly | Low-cost, single printer |
| Moreo, 2016 | No mention found | No mention found | 3 parts, glued/elastic bands | Price requirement met |
| Saldarriaga et al., 2024 | No mention found (likely fused deposition modeling) | Polylactic acid (PLA) | No mention found | Affordable, optimized design |
| Fajardo et al., 2020 | No mention found | Nylon, acrylonitrile butadiene styrene (ABS), polylactic acid (PLA) | Multi-part, elastics/strings | Less than $350, accessible |
| Nini et al., 2024 | Selective laser sintering (SLS) | No mention found | No mention found | Cost reduction via additive manufacturing |
| Zuniga et al., 2015 | Fused deposition modeling (Makerbot, Ultimaker) | Polylactic acid (PLA), acrylonitrile butadiene styrene (ABS) | Multi-part, 2.5 hours assembly | $50 materials, online files |
| Fajardo et al., 2017 | No mention found | No mention found | No mention found | Cost/time reduction |
| Cabibihan et al., 2018 | Desktop three-dimensional printing | No mention found (molds), silicone casting | Molds, casting | No mention found |
| Gu et al., 2024 | No mention found | No mention found | No mention found | No mention found |

Printing  Technology:

- Fused  deposition  modeling  or  material  extrusion:  2  studies.

- Selective  laser  sintering:  1  study.

- Desktop  three-dimensional  printing:  1  study.

- No  mention  found  of  printing  technology  in  6  studies  (including  1  described  as  "likely  fused  deposition  modeling").

Materials:

- Polylactic  acid  (PLA):  3  studies.

- Acrylonitrile  butadiene  styrene  (ABS):  2  studies.

- Nylon:  1  study.

- Three-dimensional  printed  polymers:  1  study.

- Silicone  (via  casting):  1  study.

- No  mention  found  of  materials  in  6  studies.

Assembly/Post-processing:

- Single  print  or  minimal  post-assembly:  1  study.

- Multi-part  assembly  (including  use  of  elastics,  strings,  or  glue):  3  studies.

- Molds  and  casting:  1  study.

- No  mention  found  of  assembly  or  post-processing  in  5  studies.

Cost/Accessibility:

- Statements  of  affordability,  low  cost,  price  requirement  met,  cost  reduction,  or  optimized  design:  7  studies.

- Specific  price  reported:  2  studies  ("less  than  $350"  and  "$50  materials").

- Accessibility  features  (such  as  online  files  or  use  of  a  single  printer):  3  studies.

- No  mention  found  of  cost  or  accessibility  in  2  studies.

### Data-Driven Optimization and Artificial Intelligence Applications

| Study | Artificial Intelligence Application Area | Algorithms/Methods | Input Data | Adaptation/Personalization |
| --- | --- | --- | --- | --- |
| Romero et al., 2025 | Control system (electromyography classification) | Classification | Electromyography signals | Adapts to individual users |
| Lopez, 2021 | Customization (socket modeling) | Statistical shape model (SSM), algorithm | Two-dimensional photos | No adaptation beyond initial fit |
| Moreo, 2016 | Design optimization | Statistical analysis | No mention found | Parametric personalization |
| Saldarriaga et al., 2024 | No mention found | No mention found | No mention found | No mention found |
| Fajardo et al., 2020 | Control system (electromyography) | Threshold, finite state machine | Electromyography signals | No mention found |
| Nini et al., 2024 | No mention found | No mention found | No mention found | No mention found |
| Zuniga et al., 2015 | Measurement processing | No mention found | Photographs | No mention found |
| Fajardo et al., 2017 | Control system (surface electromyography) | No mention found | Electromyography signals | No mention found |
| Cabibihan et al., 2018 | Measurement processing | No mention found | Computed tomography images | No mention found |
| Gu et al., 2024 | Design optimization | Data-driven, multi-index optimization | Motion/pressure data, clinical criteria | No mention found |

Artificial  Intelligence  Application  Area:

- Three  studies  focused  on  control  systems  using  electromyography  or  surface  electromyography  signals.

- Two  studies  addressed  design  optimization.

- Two  studies  focused  on  measurement  processing.

- One  study  focused  on  customization  (socket  modeling).

- No  mention  found  of  application  area  for  two  studies.

Algorithms/Methods  and  Input  Data:

- One  study  used  classification,  one  used  statistical  shape  modeling,  one  used  statistical  analysis,  one  used  thresholding,  one  used  finite  state  machine,  and  one  used  data-driven  multi-index  optimization.  Five  studies  did  not  mention  the  algorithm  or  method.

- Electromyography  signals  were  used  as  input  data  in  three  studies.

- Other  input  data  included  two-dimensional  photos  (one  study),  photographs  (one  study),  computed  tomography  images  (one  study),  motion/pressure  data  (one  study),  and  clinical  criteria  (one  study).  No  mention  found  of  input  data  for  three  studies.

Adaptation/Personalization:

- One  study  adapted  to  individual  users.

- One  study  used  parametric  personalization.

- One  study  reported  no  adaptation  beyond  initial  fit.

- No  mention  found  of  adaptation  or  personalization  for  seven  studies.

### Performance Validation and Functional Outcomes

| Study | Performance Metrics | Validation Method | Key Findings |
| --- | --- | --- | --- |
| Romero et al., 2025 | Bilateral Assessment Measure for Upper Limb Amputees (BAM-ULA) score (7.4/10), gross/fine motor task success | BAM-ULA protocol, 9 users | Comparable to high-end devices; 100% gross, 22.2% fine motor success |
| Lopez, 2021 | No mention found | Mechanical/user testing | Bio-inspired hand achieved superior pinch force; no quantitative data |
| Moreo, 2016 | Pinch force (35 newtons), daily activity | Mechanical/user testing | Exceeds required force; functional for daily activities; durability needs improvement |
| Saldarriaga et al., 2024 | No mention found | Preliminary evaluation, 1 user | Custom socket fabricated; no quantitative results |
| Fajardo et al., 2020 | Grip force (2.5–5 kilograms/finger, 50 newtons power), range of motion, controller response | Mechanical/user testing, National Aeronautics and Space Administration Task Load Index (NASA TLX), 10 users | High accuracy, responsive control, user-friendly, less than $350 |
| Nini et al., 2024 | Finite element method (FEM) analysis, trajectory comparison | Technical verification, 1 user | Mechanism robust; trajectory matches human finger |
| Zuniga et al., 2015 | Range of motion, user survey | Direct/photo measurement, 11 children | Positive impact on quality of life, daily use; validated measurement |
| Fajardo et al., 2017 | No mention found | Mechanical/control testing | High accuracy/responsiveness; no quantitative data |
| Cabibihan et al., 2018 | Dice similarity coefficient, percent error, mean distance | Shape/fit validation, 1 user | High accuracy (Dice similarity coefficient 0.95–0.96, less than 1% error); user satisfied |
| Gu et al., 2024 | Muscle fatigue, clinical scale scores | 5 users, load conditions | Reduced fatigue, modest score improvement |

Performance  Metrics:

- Force  or  strength  measurements:  3  studies.

- Range  of  motion:  2  studies.

- Accuracy  or  fit  metrics:  1  study.

- Task  success  (gross/fine  motor):  1  study.

- User-reported  outcomes  (such  as  surveys,  fatigue,  or  clinical  scales):  2  studies.

- Technical  or  engineering  metrics  (such  as  finite  element  method  analysis  or  trajectory  comparison):  1  study.

- No  mention  found  of  performance  metrics  in  3  studies.

Validation  Methods:

- Mechanical  testing:  4  studies.

- User  testing:  9  studies;  of  these,  3  studies  used  only  1  user,  and  the  others  used  5,  9,  10,  or  11  users.

- Technical  or  engineering  validation:  2  studies.

- Direct  or  photo  measurement:  1  study.

- Control  testing:  1  study.

- National  Aeronautics  and  Space  Administration  Task  Load  Index  (NASA  TLX):  1  study.

Key  Findings:

- Functional  success  (such  as  gross/fine  motor  task  completion  or  daily  activity):  4  studies.

- Exceeding  required  force  or  strength:  2  studies.

- High  accuracy  or  fit:  3  studies.

- User  satisfaction  or  positive  impact:  4  studies.

- Cost  information:  1  study  (device  costing  less  than  $350).

- No  quantitative  results:  3  studies.

- Durability  limitation:  1  study.

- Technical  success  (such  as  robust  mechanism  or  human-like  trajectory):  1  study.

- Comparison  to  high-end  devices:  1  study.

# References

Alejandro M. Saldarriaga, Enzo Romero, Victoria E. Abarca, and Dante A. Elías. “A Parametric Design Approach for Affordable Customized 3D Socket for Transradial Upper Limb Prostheses.” International Conference on Control, Decision and Information Technologies, 2024.

Enzo Romero, Jose G. Garcia, Magno Parra, Sebastian Caballa, Alejandro M. Saldarriaga, Edson F. Luque, Dante J. Rodriguez, Victoria E. Abarca, and Dante A. Elías. “An Affordable AI-Driven and 3D-Printed Personalized Myoelectric Prosthesis: Design, Development, and Assessment.” IEEE Access, 2025.

J. Cabibihan, M. K. Abubasha, and N. Thakor. “A Method for 3-D Printing Patient-Specific Prosthetic Arms With High Accuracy Shape and Size.” IEEE Access, 2018.

J. Lopez. “Accessible Hand Prostheses: 3D Printing Meet Smartphones,” 2021.

J. Zuniga, D. Katsavelis, Jean M Peck, John Stollberg, Marcin Petrykowski, Adam Carson, and Cristina Fernandez. “Cyborg Beast: A Low-Cost 3d-Printed Prosthetic Hand for Children with Upper-Limb Differences.” BMC Research Notes, 2015.

Julio Fajardo, Victor Ferman, Ali Lemus, and E. Rohmer. “An Affordable Open-Source Multifunctional Upper-Limb Prosthesis with Intrinsic Actuation.” Workshop on Advanced Robotics and Its Social Impacts, 2017.

Julio Fajardo, Victor Ferman, Diego Cardona, Guillermo Maldonado, Ali Lemus, and E. Rohmer. “Galileo Hand: An Anthropomorphic and Affordable Upper-Limb Prosthesis.” IEEE Access, 2020.

Ludovica Nini, Alessandro Ceccarelli, N. Tagliamonte, Loredana Zollo, and Fabrizio Taffoni. “Parametric 3D Modeling of a Customized Prosthetic Hand Finger for Additive Manufacturing.” International Conference on Biomedical Robotics and Biomechatronics, 2024.

M. Moreo. “Parametric Design of a 3D Printable Hand Prosthesis for Children in Developing Countries,” 2016.

Yu Gu, Long He, Haozhou Zeng, Jiaxing Li, Ning Zhang, Xiufeng Zhang, and Tao Liu. “A Data-Driven Design Framework for Structural Optimization to Enhance Wearing Adaptability of Prosthetic Hands.” IEEE Transactions on Neural Systems and Rehabilitation Engineering, 2024.